Specification

4G LTE WiFi Gigabit Router

Plug & Play

Model: BT44i

2X2 MIMO

T O R N A D O

Maintenance – Free Connections

Introduction

LTE (Long Term Evolution) is already gaining momentum as the world’s most rapidly deployed cellular technology, giving mobile wireless broadband services to millions of users worldwide. Consumers are increasingly looking for always on, always connected mobile experience delivering high data rate services on small factor mobile devices whilst at the same time expecting long battery life to minimize recharge cycles.

BT44i is Cat4 and 4G LTE indoor multi-service product solution for Enterprise and SHSO application. Specifically designed to meet integrated data, and dual bands Wi-Fi access needs for residential, business and home users.

In this product, we provide Gigabit networking and single bands Wi-Fi AP functionalities, it enables wide service coverage and provides high data throughput and networking features to customers who need easy broadband access and hot-spot Wi-Fi connectivity.

Technical Features

- Support 4 LAN ports, 1GE and include 1 WAN port

- Support Wi-Fi 2.4GHz Concurrent

- Support DMZ host Configuration

- Support Short Message System

- Support CWMP TR069 and Certification

- Support Server Network Time protocol & Time Synchronization (SNTP).

- Support EARFCN lock, PCI lock, PLMN lock, ICCID lock/

- Support two login access, root for administrator and admin for clients

- Support DDNS Agency

- Support WPS Soft Button and Push Button

BASIC SPECIFICATIONS

| Main Chipset | Quectel EC200AELV1 |
| --- | --- |
| LTE Standard | 3GPP latest Release, 8, 9, 10 & 11 |
| Ethernet LAN/WAN Port | 4*RJ-45 port LAN 10/100 auto-sensing, |
| Smart Wi-Fi Dual Mode | 2.4GHz (802.11 b/g/n and MIX) |
| LED Indicators | 8 Blue LED indicators (PWR, Wi-Fi 2.4GHz, LTE, RF and 4 Signal Level) |
| USIM | 1.8v/3.0v Mini-SIM (2FF) |
| Antenna | PCB Built-in high gain, 2.4GHz |
| Restore Button | Press for 10 Sec. to factory restore settings |
| Power Switch | ON/OFF Button |
| WPS Switch | ON/OFF Push button 10 Sec. hold |
| Power Supply | Input: Universal range 100V to 240V AC, Output 12VDC / 2A |

LTE SPECIFICATIONS

| Duplex Mode | FDD & TDD |
| --- | --- |
| Frequencies Bands | 4G LTE: B1, B3, B7, B8, B20, B38, B40, B41 3G WCDMA: B1, B5, B8 |
| Bandwidth | 5, 10, 15, 20MHz |
| MIMO | 2x2 MIMO |
| Max Output Power | 23 ± 2dBm per Tx antenna |
| Tx / Rx | 1T2R configurations |
| Uplink Channel | PUCCH, PUSCH, PRACH |
| Downlink Channel | PDCCH, PDSCH, PBCH, PCFICH, PHICH |
| Peak Rate (20 MHz) | FDD: DL 150 Mbps, UL 50 Mbps TDD: DL 130 Mbps, UL 30 Mbps |
| Number of Resource Blocks | 6 ~ 110 |
| Transmission Mode | TM 1.2,3,4,7,8,9 |
| Modulation | DL: QPSK, 16QAM, 64QAM UL: QPSK, 16QAM |
| Receive Sensitivity | -96±2 dBm @QPSK, 20MHz, 25°C, -96±2 dBm @ 2RX Diversity |
| Antenna Type | PCB |
| Antenna Gain | 5dBi |
| Antenna Efficiency | >60% or more |
| Polarization | Linear, Vertical |
| VSWR | 700~2700＜4 and 3300~3800＜3 |
| Radiation | Omni-directional |
| Admitted Power | 1w |

LTE RADIO STATUS INFORATION

| General Info | USIM | Operator | IMEI |
| --- | --- | --- | --- |
|  | PLMN | eNodeB ID | IMSI |
| Signal Details | PCI | EARFCN | Bandwidth |
|  | SINR | Band | Cell ID |
|  | RSSI | RSRQ | RSRP |

SOFTWARE SPECIFICATIONS

| LTE Network Mode | - NAT Mode |
| --- | --- |
| LAN Network | - IPv4 address - DHCP Statics Lease - DHCP Server, Client |
| Mobile Network Connection | - Auto - Manual |
| 4G LTE Connection | - Roam (Enable / Disable) - Power Scan (First Detected Cell, Strongest Cell) - Dedicated Mode: PCI, EARFCN, Frequency & Cell - APN Management Settings |
| Wi-Fi (2.4GHz) | - Wi-Fi Enable / Disable - Bandwidth: 20MHz / 40MHz - Access Point Isolate - Channel Selection: Manual, Auto - Channel Range 1 ~ 11 - SSID: Hidden / Disable / Edit - Security type: No Authentication, WPA-PSK, WPA2-PSK, WPA/WPA2-PSK, - WPA Algorithm: (TKIP/AES) - WPS Enable / Disable - Country Code: Lock IRAN |
| WAN Network | - WAN (LTE) - WAN (Input Access) |
| Data Services | - Port Range Forwarding - Port Range Triggering - DMZ Setting |
| Remote Access | - DDNS or DNS Agent |
| Security | - Terminal Firewall - IPv4 Filter - MAC Filter - WAN Filter - Port Triggering Filter - Port Forwarding Filter |
| System | - Remote Web Login - Access Control - Ping Watchdog |
| Terminal Security | - SSH - Telnet |
| CWMP | - Primary TR069 - Certificated Primary |
| Network Diagnostics | - Ping - Traceroute |
| Administration | - Web Protocols (https 443 and http 80) - NTP Setting - MTU Setting - PIN Management - USIM Setting - ICCID Setting |
| Statistics | - LTE Status - System up time - Device Status - Firewall Status - Roaming Status |
| Network Users | - DHCP Client List - Wi-Fi 2.4GHz Client List |
| Maintenance | - Date and time setting - Restore Factory Settings - Backup Configuration file - Firmware Upgrade locally Web UI |
| Other Applications | - SMS Sent / Receive - WPS Soft Button |

Wi-Fi SPECIFICATIONS

| Standards Supported | - 802.11b/g/n compatible Wi-Fi - 802.11e QoS Enhancement (WMM) - 802.11i (WPA, WPA2) No Authentication, shared key, and pair-wise key authentication services |
| --- | --- |
| Channel Bandwidth | 20MHz, 40MHz |
| Frequency 2.4GHz | 2.412 (Channel 1) ~ 2.462 (Channel 11) |
| TXRX | 1T2R |
| MIMO | 2x2 MIMO |
| Maximum Data Rate | - 802.11b: 11Mbps - 802.11g: 54Mbps - 802.11n: 300Mbps |
| Modulation | - DSSS with DBPSK and DQPSK, CCK - OFDM with BPSK, QPSK, 16QAM |
| Convolutional Coding Rate: | 1/2, 2/3, 3/4, and 5/6 |
| Receive Sensitivity | - -64dBm @ 65Mbps, typical for 802.11n - -65dBm @ 54Mbps, typical for 802.11g - -76dBm @ 11Mbps, typical for 802.11b |
| Maximum Output Power | - 20dBm for ODFM & 18dBm for CCK modulation |
| Fast Receiver | Automatic Gain Control (AGC) |
| Antenna Type | Build-in separate antenna, 2.4GHz |
| Antenna Gain | 2.4GHz / 3dBi |

ENVIRONMENTAL SPECIFICATIONS

| Operating Temperature | -40°C to 55°C |
| --- | --- |
| Storage Temperature | -40°C to 70°C |
| Operating Humidity | 5% to 95% |
| Ingress Protection Rating | IP65 |

PACKAGE DELIVER

|  | Assembled Product |
| --- | --- |
|  | Ethernet Cable, Cat5 |
|  | 12V/2A AC / DC Adaptor |
|  | Quick Installer Guide |

Contact Information

Manufacture: crikleco Company

Address: AI Moosa Tower 2, Trade Center First, Dubai,UAE

EMail: info@crikleco.com