Specification

4.5G LTE Wi-Fi Gigabit Router

Plug & Play

Model: CK62i Pro

2CA + 4x4 MIMO

W h i t e   T O R N A D O

Maintenance – Free Connections

Introduction

LTE (Long Term Evolution) is already gaining momentum as the world’s most rapidly deployed cellular technology, giving mobile wireless broadband services to millions of users worldwide. Consumers are increasingly looking for always on, always connected mobile experience delivering high data rate services on small form factor mobile devices whilst at the same time expecting long battery life to minimize recharge cycles.

CK62i Pro is Cat6 and 4.5G LTE indoor multi-service product solution for Enterprise and SHSO application. Specifically designed to meet integrated data, and dual bands Wi-Fi access needs for residential, business and enterprise users.

In this product, we provide Gigabit networking and dual bands Wi-Fi AP functionalities, it enables wide service coverage and provides high data throughput and networking features to customers who needs easy broadband access, hot-spot Wi-Fi connectivity.

Interactive User Interface

Technical Features

- Support Dual Mode LTE-TDD / LTE-FDD

- Support Dual Band Wi-Fi (2.4GHz / 5GHz) Concurrent

- Support Dual Stack IPv4 / IPv6

- Support Carrier Aggregation Combination (Inter / Intra)

- Support Category 6 (300Mbps Down Link, 50Mbps Up Link) DL MIMO 4*4

- Support 2 LAN ports, 1GE

- Support 1 WAN port

- Support Multi APN Management

- Support Multi SSID

- Support Short Message System

- Support Small Network Management Protocol (SNMP)

- Support CWMP TR069 and Certification (Primary and Secondary)

- Support Server Network Time protocol (SNTP).

- Support EARFCN lock, frequency lock, and PCI lock, PLMN lock, ICCID lock/

- Support two login access, root for administrator and admin for clients

- Support DDNS or DNS Agency

- Support WPS Soft Button

- Support Routing (NAT, Tunnel, and Bridge Mode)

- Support VPN (L2TP, GRE, PPTP, VXLAN)

- Support IPSec VPN

- Support Wi-Fi-Dog Solution (Optional)

BASIC SPECIFICATIONS

| LTE Standard | 3GPP latest Release, 8, 9, 10 & 11 |
| --- | --- |
| Ethernet LAN/WAN Port | 1*RJ-45 port LAN 10/100/1000 auto-sensing, 1*RJ-45 port LAN / WAN |
| Smart Wi-Fi Dual Band | 2.4GHz (802.11 b/g/n and MIX) 5GHz (802.11 ac) |
| LED Indicators | 8 Blue LED indicators (PWR, Wi-Fi 2.4GHz, Wi-Fi 5GHz, LTE, RF and 4 Signal Level) |
| USIM | 1.8v/3.0v Mini-SIM (2FF) |
| RAM / Flash | 1Gb / 2Gb |
| Antenna | Wi-Fi: 2 Separate PCB Built-in high gain, 2.4GHz, 5GHz LTE: 4 Separate internal 5dBi |
| Restore Button | Press for 10 Sec. to factory restore settings |
| Power Switch | ON/OFF Button |
| Wi-Fi Switch | ON/OFF Push button 15 Sec. hold |
| WPS Switch | ON/OFF Push button 10 Sec. hold |
| Front Panel LED | ON/OFF Push button |
| Power Supply | Input: Universal range 100V to 240V AC, Output 12VDC / 2A |
| Power Bank | Chargeable Battery and charging system (Optional) |

LTE SPECIFICATIONS

| Duplex Mode | Dual Mode (TDD Only, FDD Only TDD/FDD Mixed) |
| --- | --- |
| Support Bands | FDD Band: B1, B3, B7, B8, B20 TDD Band: B40, B42 |
| Bandwidth | 1.4, 3, 5, 10, 15, 20MHz |
| Carrier Aggregation & MIMO | 2CC CA [Inter / Intra] DL 2CA (2x2 MIMO 64 QAM) DL 4x4 MIMO 64 QAM |
| Max Output Power | 23 ± 2dBm per Tx antenna |
| Tx / Rx | 1Tx / 4Rx |
| Uplink channel | PUCCH, PUSCH, PRACH |
| Downlink channel | PDCCH, PDSCH, PBCH, PCFICH, PHICH |
| Peak Rate (20 MHz) | FDD: DL 300 Mbps, UL 100 Mbps TDD: DL 220 Mbps, UL 15 Mbps |
| Number of Resource Blocks | 6 ~ 110 |
| Transmission Mode | TM 1.2,3,4,7,8,9 |
| Modulation | DL: QPSK, 16QAM, 64QAM UL: QPSK, 16QAM, 64QAM |
| Receive Sensitivity | -94dBm @ QPSK, 20 MHz, 25°C |
| Antenna Type | ABS |
| Antenna Gain | 5dBi |
| Antenna Efficiency | >60% or more |
| Polarization | Linear, Vertical |
| VSWR | 700~2700＜4 and 3300~3800＜3 |
| Radiation | Omni-directional |
| Admitted Power | 1w |

LTE RADIO STATUS INFORATION

| General Info | USIM | Operator | EMM State |
| --- | --- | --- | --- |
|  | PLMN | eNodeB ID | RRC State |
|  | ICCID | Cell ID | TAC |
|  | IMSI | C-RNTI | ECI |
| Signal Details & Carrier Aggregation | PCI | EARFCN | Bandwidth |
|  | SINR | CINR | CQI |
|  | RSSI | RSRQ | RSRP |
|  | DL MCS | UL MCS | RANK |
|  | DL Frequency | DL Frequency | Band |
|  | Transfer mode | Intra Band | Inter Band |
|  | TXPWR | PCC | SCC (0,1,2,3) |

SOFTWARE SPECIFICATIONS

| LTE Network Mode | - NAT Mode - Bridge Mode - Tunnel Mode |
| --- | --- |
| LAN Network | - IPv4 address - DHCP Statics Lease - DHCP Relay - DHCP Server, Client |
| Mobile Network Connection | - Auto - Manual |
| 4G LTE Connection | - Roam (Enable / Disable) - Power Scan (First Detected Cell, Strongest Cell) - Duplex Selection: (FDD/TDD Mixed), TDD Only. FDD Only - Dedicated Mode: PCI, EARFCN, Frequency & Cell - Cell Scan - Neighbor Cell Auto Scan - APN Management Settings |
| Wi-Fi (2.4GHz) | - Wi-Fi (Enable / Disable) - Frequency Range: 2.412GHz ~ 2.462GHz - Bandwidth: 20MHz / 40MHz - Access Point Isolate - Channel Mode: Manual, Auto - Channel Range 1 ~ 13 (Europe) - SSID: Hidden / Disable / Edit, Support four SSID - Security type: No Authentication, WPA-PSK, WPA2-PSK, WPA/WPA2-PSK - WPA Algorithm: (TKIP/AES) - WPS Enable / Disable - Country Code: IRAN |
| Wi-Fi (5GHz) | - Wi-Fi (Enable / Disable) - Frequency Range: 5.18GHz ~ 5.7GHz - Bandwidth: 20MHz / 40MHz / 80MHz (Auto) - Access Point Isolate - Channel Mode: Manual, Auto - Channel Range 36 ~ 64 (Europe) - SSID: Hidden / Disable / Edit, - Security type: No Authentication, WPA-PSK, WPA2-PSK, WPA/WPA2-PSK, WPA3-PSK - WPA Algorithm: (TKIP/AES) - Country Code: IRAN |
| WAN Network | - WAN (LTE) - WAN (Input Access) |
| VPN Server | - IPsec - L2TPv2 & v3 - GRE - PPTP - VxLAN L2/L3 |
| Data Services | - Port Range Forwarding - Port Range Triggering - DMZ Setting - UPnP Setting - ALG Setting |
| Remote Access | - DDNS or DNS Agent - SNMP |
| Firewall | - Terminal Firewall (Enable / Disable) - IPv4/IPv6 Filter (Black List / white List) - MAC Filter (Black List / white List) - URL Filter (Black List / white List) |
| System Security | - Remote Web Login - Access Control - Block Port scanner - Block SYN Flood - SPI Firewall |
| Terminal Security | - SSH - Telnet |
| Device Management | - SNMP v3 - SAS |
| CWMP | - Primary TR069 - Certificated Primary - Secondary TR069 - Certificated Secondary |
| Network Diagnostics | - TCP Dump - Ping - Traceroute - iPerf |
| Administration | - Web Protocols (https 443 and http 80) - NTP Setting - MTU Setting - PIN Management - USIM Setting - ICCID Setting |
| Statistics | - LTE Status - System up time - Device Status - Firewall Status - Roaming Status - Carrier Aggregation Status |
| Network Users | - DHCP Client List - Wi-Fi 2.4GHz Client List - Wi-Fi 5GHz Client List |
| Maintenance | - Date and time setting - Restore Factory Settings - Backup Configuration file - Firmware Upgrade locally Web UI - Firmware Over The Air (FOTA) - Ping Wi-Fi dog Setting |
| Routing | - Current Route Status - Routing Map (ARP / Active Route) - Static Routes (LAN / WAN / L3 VPN) - Ping Watchdog |
| System Tools | - Operating Log (Info, Warnings, Errors, Critical) - Run-time Log (Info, Warnings, Errors, Critical) - Filter, Select, Display, export Logs |
| Debugging Software | - DM tools for troubleshooting |
| Other Applications | - WiFi-Dog Solution - SMS Sent / Receive - Calendar - Events (IP Address Block, Reboot, WiFi, LTE) - WPS Soft Button - SSID Bind APN |

Wi-Fi SPECIFICATIONS

| Standards Supported | - 802.11b/g/n compatible Wi-Fi - 802.11e QoS Enhancement (WMM) - 802.11i (WPA, WPA2) No Authentication, shared key, and pair-wise key authentication services |
| --- | --- |
| Channel Bandwidth | 20MHz, 40MHz, 80MHz |
| Frequency 2.4GHz | 2.412 (Channel 1) ~ 2.472 (Channel 13) |
| Frequency 5GHz | 5.18 (Channel 36) ~ 5.32 (Channel 64) |
| TXRX | 2T2R |
| MIMO | 2x2 MIMO |
| Maximum Data Rate | - 802.11b: 11Mbps - 802.11g: 54Mbps - 802.11n: 300Mbps - 802.11ac: 866Mbps |
| Modulation | - DSSS with DBPSK and DQPSK, CCK - OFDM with BPSK, QPSK, 16QAM, and 64QAM |
| Convolutional Coding Rate: | 1/2, 2/3, 3/4, and 5/6 |
| Receive Sensitivity | - -51dBm @ 135Mbps, typical for 802.11ac - -64dBm @ 65Mbps, typical for 802.11n - -65dBm @ 54Mbps, typical for 802.11g - -76dBm @ 11Mbps, typical for 802.11b |
| Maximum Output Power | - 2.4GHz / 20dBm for ODFM & 18dBm for CCK modulation - 5GHz / 23dBm |
| Fast Receiver | Automatic Gain Control (AGC) |
| Antenna Type | Build-in separate antenna, 2.4GHz, 5GHz |
| Antenna Gain | 2.4GHz / 3dBi 5GHz / 5dBi |

ENVIRONMENTAL SPECIFICATIONS

| Operating Temperature | -40°C to 55°C |
| --- | --- |
| Storage Temperature | -40°C to 70°C |
| Operating Humidity | 5% to 95% |
| Ingress Protection Rating | IP65 |

PACKAGE DELIVER

| ITEMS | Assembled Product |
| --- | --- |
|  | Ethernet Cable, Cat5 |
|  | 12V-2A AC / DC Adaptor |
|  | Quick Installer Guide |

Contact Information

Manufacture: crikleco Company

Address: AI Moosa Tower 2, Trade Center First, Dubai,UAE

EMail: info@crikleco.com