Specification

GPON Wi-Fi Gigabit Router

Smart TV BOX

Model: BTQ43i

S M A R T   F I B E R   T O R N A D O

Maintenance – Free Connections

Introduction

BTQ43i is for broadband access market based on the introduction of EPON & GPON technology passive optical network terminal products. It’s with EPON/GPON OLT are used together to provide a complete broadband access solution.

This model can integrate wireless function with meeting 802.11 n/b/g/ac technical standards, It has built‐in high gain directional antenna, the wireless transmission rate up to 866Mbps. It has the characteristics of strong penetrating power and wide coverage. It can provide users with more efficient data transmission security.

BTQ43i is highly advanced GPON indoor multi-service product solution for Enterprise and SHSO application. Specifically designed to meet integrated data, and dual bands Wi-Fi access needs for residential, business and enterprise users.

In this product, we provide Gigabit networking and dual bands Wi-Fi AP functionalities, it enables wide service coverage and provides high data throughput and networking features to customers who needs easy broadband access, hot-spot Wi-Fi connectivity.

The device also have Smart TV Box, VoIP and USB interface support.

Technical Features

- Support Dual Band Wi-Fi (2.4GHz / 5GHz) Concurrent

- Support Dual Stack IPv4 / IPv6

- Support 3 LAN ports, 1GE

- Support 1 WAN port

- Support Small Network Management Protocol (SNMP)

- Support CWMP TR069 and Certification (Primary and Secondary)

- Support Server Network Time protocol (SNTP).

- Support two login access, root for administrator and admin for clients

- Support DDNS or DNS Agency

- Support WPS Soft Button

- Support Routing (NAT, Tunnel, and Bridge Mode)

- Support VPN (L2TP, GRE, PPTP)

- Support IPSec VPN

GPON Specfications

| Standard | FSAN G.984.2, Class B+ |
| --- | --- |
| Authentication | Consistent with the SN or password |
| Downstream | 2.488Gbps |
| Upstream | 1.244Gbps |
| Receiver Sensitivity | -28dBm |
| Transmit Optical Power | 0.5~5dBm |
| Saturation Optical Power | -8dBm |
| Wavelength Transmitter | 1310nm |
| Wavelength Receiver | 1490nm |

EPON Specfications

| Standard | 1000BASE-PX20+ Symmetric CTC2.1, CTC3.0 |
| --- | --- |
| Authentication | Consistent with the MAC or LOID |
| Downstream | 1.25Gbps |
| Upstream | 1.25Gbps |
| Receiver Sensitivity | -27dBm |
| Transmit Optical Power | 0~4dBM |
| Saturation Optical Power | -3dBm |
| Wavelength Transmitter | 1310nm |
| Wavelength Receiver | 1490nm |

ONT SPECIFICATIONS

| GPON Standard | ITU-T G.984.X |
| --- | --- |
| Dimensions (LxWxH) | 265mm x 16mm x 32mm |
| Weight | TDB |
| Wi-Fi Dual Band | 2.4GHz (802.11 b/g/n and MIX) 5GHz (802.11 ac) |
| Wi-Fi Antenna | 2 Separate PCB Built-in high gain, 2.4GHz, 5GHz |
| LED Indicators | WPS٫WLAN٫LAN4٫LAN3٫LAN2٫LAN1٫USB٫TEL٫LOS٫PON٫ONT٫OTT |
| UNI | 1POT + 3GE + 2.4G/5G Wi-Fi + 3USB + 1HDMI + 1AV OUT + 1IR Receiver |
| Restore Button | Press for 10 Sec. to factory restore settings |
| ONT Power Switch | ON/OFF Button |
| OTT Power Switch | ON/OFF Button |
| Power Supply | Input: Universal range 100V to 240V AC, Output DC 12V/3A |
| Static power consumption | 3W |
| Maximum power consumption | 10W |
| Installation mode | DESKTOP, WALL MOUNT |

port specfications

| Fiber Port | SC/PC single-mode fiber |
| --- | --- |
|  | Flexible mapping between GEM Port and TCONT |
|  | Bi-directional FEC |
|  | SR-DBA and NSR-DBA |
|  | Type B (Single-homing & Dual-homing) |
| Ethernet Port | Ethernet port-based VLAN tags and tag removal |
|  | 1:1 VLAN, N:1 VLAN or VLAN transparent transmission |
|  | QinQ VLAN |
|  | Limit on the number of learned MAC addresses |
|  | Auto-adaptive 10 Mbit/s, 100 Mbit/s or 1000 Mbit/s |
|  | Full/half duplex mode, auto MDI/MDI-X |
| POTS Port (Optional) | Maximum REN:4 |
|  | G.711A/µ, G.729a/b & G.722 encoding/decoding |
|  | T.30/T.38/G.711 Fax mode |
|  | DTMF, Emergency calls (with the SIP protocol) |
| USB Port | USB 2.0 interface |
|  | Transmission Rate 480Mbps |
|  | FTP-based network storage |
|  | File/Print sharing basted on SAMRA |
|  | DLNA function |
| WLAN | Comply with IEEE 802.11a/b/g/n/ac protocol standard |
|  | 5dBi antenna gain |
|  | Maximum ideal rate up to 1.2Gbps |
|  | 2.4GHz supports 2*2 MIMO technology |
|  | 5GHz supports 2*2 MIMO technology |
|  | WMM/Multiple SSIDs/WPS |
| HDMI Port | 4K*2K UHD Output, HDMI 2.0A |
| AV Port | 480i/576i standard definition output |
|  | Remote Receiver Connection |

SOFTWARE SPECIFICATIONS

| LAN Network | - IPv4 address - DHCP Statics Lease - DHCP Relay - DHCP Server, Client |
| --- | --- |
| Wi-Fi (2.4GHz) | - Wi-Fi (Enable / Disable) - Frequency Range: 2.412GHz ~ 2.462GHz - Bandwidth: 20MHz / 40MHz - Access Point Isolate - Channel Mode: Manual, Auto - Channel Range 1 ~ 13 (Europe) - SSID: Hidden / Disable / Edit, Support four SSID - Security type: No Authentication, WPA-PSK, WPA2-PSK, WPA/WPA2-PSK, WPA3-PSK - WPA Algorithm: (TKIP/AES) - WPS Enable / Disable - Country Code: IRAN |
| Wi-Fi (5GHz) | - Wi-Fi (Enable / Disable) - Frequency Range: 5.18GHz ~ 5.7GHz - Bandwidth: 20MHz / 40MHz / 80MHz (Auto) - Access Point Isolate - Channel Mode: Manual, Auto - Channel Range 36 ~ 64 (Europe) - SSID: Hidden / Disable / Edit, - Security type: No Authentication, WPA-PSK, WPA2-PSK, WPA/WPA2-PSK, WPA3-PSK - WPA Algorithm: (TKIP/AES) - Country Code: IRAN |
| WAN Network | - WAN (Input Access) |
| VPN Server | - IPsec - L2TPv2 & v3 - PPTP |
| Data Services | - Port Range Forwarding - Port Range Triggering - DMZ Setting - UPnP Setting - ALG Setting |
| Remote Access | - DDNS or DNS Agent - SNMP |
| Firewall | - Terminal Firewall (Enable / Disable) - IPv4/IPv6 Filter (Black List / white List) - MAC Filter (Black List / white List) - URL Filter (Black List / white List) |
| System Security | - Remote Web Login - Access Control - Block Port scanner - SPI Firewall |
| Terminal Security | - SSH - Telnet |
| Device Management | - SNMP v3 - SAMBA Service |
| CWMP | - Primary TR069 |
| Network Diagnostics | - Ping - Traceroute |
| Administration | - Web Protocols (https 443 and http 80) - NTP Setting |
| Network Users | - DHCP Client List - Wi-Fi 2.4GHz Client List - Wi-Fi 5GHz Client List |
| Maintenance | - Date and time setting - Restore Factory Settings - Backup Configuration file - Firmware Upgrade locally Web UI - Firmware Over The Air (FOTA) |

Wi-Fi SPECIFICATIONS

| Standards Supported | - 802.11b/g/n compatible Wi-Fi - 802.11e QoS Enhancement (WMM) - 802.11i (WPA, WPA2) No Authentication, shared key, and pair-wise key authentication services |
| --- | --- |
| Channel Bandwidth | 20MHz, 40MHz, 80MHz |
| Frequency 2.4GHz | 2.412 (Channel 1) ~ 2.472 (Channel 13) |
| Frequency 5GHz | 5.18 (Channel 36) ~ 5.32 (Channel 64) |
| TXRX | 2T2R |
| MIMO | 2x2 MIMO |
| Maximum Data Rate | - 802.11b: 11Mbps - 802.11g: 54Mbps - 802.11n: 300Mbps - 802.11ac: 866Mbps |
| Modulation | - DSSS with DBPSK and DQPSK, CCK - OFDM with BPSK, QPSK, 16QAM, and 64QAM |
| Convolutional Coding Rate: | 1/2, 2/3, 3/4, and 5/6 |
| Receive Sensitivity | - -51dBm @ 135Mbps, typical for 802.11ac - -64dBm @ 65Mbps, typical for 802.11n - -65dBm @ 54Mbps, typical for 802.11g - -76dBm @ 11Mbps, typical for 802.11b |
| Maximum Output Power | - 2.4GHz / 20dBm for ODFM & 18dBm for CCK modulation - 5GHz / 23dBm |
| Fast Receiver | Automatic Gain Control (AGC) |
| Antenna Type | Build-in separate antenna, 2.4GHz, 5GHz |
| Antenna Gain | 2.4GHz / 3dBi 5GHz / 5dBi |

OTT SPECIFICATIONS

| Video |  | All winner Video Engine (AVE) with dedicated hardware decoders and encoders |
| --- | --- | --- |
|  |  | Support multi-video decoder up to 4K@60fps |
|  |  | Supports multiple "secured" video decoding sessions and simultaneous decoding and encoding |
|  |  | Video/Picture Decoding |
| Video/Picture Decoding |  | VP9 Profile-2 up to 4K@30fps |
|  |  | H.265 Mam10@1.5.1 up to 4K@60fps |
|  |  | AVS2 JiZhum 10bit Profile up to 4K@60fps |
|  |  | H.264 BP/MP/HP@L4.2 up to 4K@30fps |
|  |  | H.263 BP up to 1080P@60fps |
|  |  | MPEG-4 SP/ASP@L5 up to 1080P@60fps |
|  |  | MPEG-2 MP/HI up to 1080P@60fps |
|  |  | MPEG-1 MP/HI up to 1080P@60fps |
|  |  | Xvid ap to 1080P@60fps |
|  |  | Sorenson Spark up to 1080P@60fps |
|  |  | VP8 up to 1080P@60fps |
|  |  | AVS/AVS+ JiZhun Profile up to 1080P@60fps |
|  |  | WMV9٫VCI SP/MP/AP up to 1080P@60fps |
|  |  | JPEG IFIF file format up to 45M PPS |
|  |  | Multiple language and multiple format sub-title video support |
|  |  | MJPEG and JPEG unlimited pixel resolution decoding (ISO/IEC-10918) |
|  |  | Supports JPEG thumbnail, scaling, rotation and transition effects |
|  |  | Supports *mkv. *.wmv, *.mpg*.mpeg, *.dat, *.avi, *mov, *iso, *.mp4, *.rm and ".jpg file formats. |
| Image |  | HD JPEG, BMP GIF. PNG. TIF |
| language |  | English French German Spanish Italian etc. multilateral languages |
| OS |  | Android 10.0 |
| Audio |  | Supports MP3, AAC, WMA, RM, FLAC, Ogg and programmable with 7.1/5.1 down-mixing |
|  |  | Built-in serial digital audio SPDIF/IEC958 input/output and PCM input/output |
|  |  | Two DAC channels |
|  |  | Supports 1 audio output interface (differential LINEOUTP/N or single-end LINEOUTL/LINFOUTR) |
|  |  | One Audio HUB, supporting internal mixing function |
|  |  | Embedded 3 I2S/PCM (12S0 for extended audio codec. 12S2 for BT, 12S3 for digital power amplifier) |
|  |  | Supports Left-justified, Right-justified, Standard 12S mode. PCM mode, and TDM mode |
|  |  | 125 mode supports 8 channels, and 32-bit/192kbit sample rate |
|  |  | 125 and TDM modes support maximum 16 channels, and 32- bit٫96kbit sample rate |
|  |  | One OWA OUT interface, supporting 16-/20-/24-bit inputs and outputs |
|  |  | Integrated digital microphone (DMIC), supporting maximum 8 digital PDM microphones |
| Applications | Online | Browse all video websites, support Netflix, Hulu, Flixster, Youtube, etc. |
|  | Apps | Apps download freely form android market amazon app store etc. |
|  | Talk online | Support SKYPE video call MSN, facebook, Twitter. QQ etc. |
|  | Medium | Local Media playback. Support HDD, U Disck, TF Card. |
|  | Others | Support Email, office suit etc. |
|  |  | Support DLNA function |
|  |  | Support 2.4G wireless mouse/keyboard |

ENVIRONMENTAL SPECIFICATIONS

| Operating Temperature | 0℃~45℃ |
| --- | --- |
| Standby Temperature | -40℃~70℃ |
| Operating Humidity | 10% - 90% |
| Standby Humidity | 5% - 90% RH |

PACKAGE DELIVER

|  | Assembled Product |
| --- | --- |
|  | Quick Installer Guide |
|  | Remote Controller |
|  | Ethernet Cable, Cat5 |
|  | HDMI Cable |
|  | 12V-3A AC / DC Adaptor |
|  | 2 x AAA 1.5v Battery |
|  | SC – SC patch cord |

Contact Information

Manufacture: crikleco Company

Address: AI Moosa Tower 2, Trade Center First, Dubai,UAE

EMail: info@crikleco.com